# Inference Optimization Experiments

Qwen3-14B and Qwen3-30B-A3B on NVIDIA GB10 — lossless and lossy serving configuration search

GB10 (Blackwell sm121, 122 GB unified, 273 GB/s) · vLLM 0.26.0 · torch 2.11.0+cu130 · August–September 2026

# Summary

Two models were searched against the same hardware, workload trace and SLO. Each search starts from an unoptimized deployment and proceeds through a LOSSLESS stage, which changes launch flags only and cannot move quality, and a LOSSY stage, which rewrites the weights and can.

On Qwen3-14B the lossless stage alone gave 4.0x goodput at zero measurable accuracy cost. Weight quantization to NVFP4 reached 26x over stock, at 10.6 GB instead of 29.5 GB.

Qwen3-30B-A3B — twice the parameters, 3.4B active per token — starts 5.2x ahead of the 14B at seed and reaches 2.24x through its own lossless search. Optimized, the two land within 1.22x of each other: the search is worth far more on the worse-starting configuration.

The MoE is limited by PREFILL, not decode. Its decode floor is 4.4x better than the 14B's, but a prompt routes across effectively all 128 experts, so prefill reads the full 61 GB. Its optimized peak sits at 8 concurrent requests; the seed config sustained only 2.

MATH-500 accuracy is unchanged across every lossless step on both models, and across most lossy steps. The one real loss found — NVFP4 at −0.0253 — was only resolvable at n=500.

# What was measured

|  | value |
| --- | --- |
| Models | Qwen/Qwen3-14B — 14.8 B dense, GQA, bf16, 29.5 GB weights Qwen/Qwen3-30B-A3B — 30.5 B total / 3.4 B active, 128 experts (8 routed), 61.1 GB resident, 6.7 GB active |
| Hardware | NVIDIA GB10 ×1 — sm121, 121.7 GB UNIFIED memory, 273 GB/s, FP8+FP4 |
| Workload | data/trace_shared.jsonl — 800 requests, mean input 620 tok (p99 2660), mean output 260 tok (p99 804), 16.0 req/s, 31.4% prefix overlap |
| SLO | TTFT p99 ≤ 500 ms, ITL p99 ≤ 250 ms |
| Demand | 4,159 tok/s of on-time output (16.02 req/s × 259.6 tokens). Replica counts are ceil(demand ÷ goodput). |
| Quality | MATH-500, exact match on the boxed answer, 3 repeats. RULER is EXCLUDED — see Caveats. |

Metrics

goodput (tok/s and req/s) — output counted ONLY from requests that met the SLO. The headline number: throughput that misses the latency target is not capacity.

throughput (tok/s and req/s) — all output, SLO or not. The gap between throughput and goodput is work the server did but could not sell.

L — in-flight requests. NOT a tuning knob: by Little's Law (L = λW) it is an outcome of arrival rate and service time. Traversals SWEEP it and report the peak; ladder runs PIN it.

spread — largest minus smallest of three identical repeats. The measurement's own resolution; a quality delta smaller than it is not a finding in either direction.

# Two instruments, and why rows do not mix

Numbers in this document come from two different measurements. They answer different questions and must not be compared across the line.

|  | traversal (run.py optimize) | ladder (eval_repro.py) |
| --- | --- | --- |
| operating point | SWEPT — brackets {L/2, L, 2L}, scores the PEAK, extends if the peak lands on an edge | PINNED at one L for every row |
| loop | closed — holds L in flight | open — fixed arrival rate |
| answers | what is this config's capacity? | how do these configs compare at one point? |
| used for | the lossless searches on both models | the lossy ladders on Qwen3-14B, and the stock baselines |

The practical consequence is that the ladder rows are a LOWER BOUND. They were pinned at L=30, chosen for the bf16 baseline, and the traversals later showed that no configuration on this hardware sustains anything near that: the 14B peaks around 30 but the MoE collapses above 8. A row measured above its own capacity reports the cliff, not the config.

# Experiment 1 — Qwen3-14B, lossless search

run.py optimize, swept operating point, 10 launches. Every percentage is against the stage 1.3 seed.

| node | kept | goodput tok/s | vs seed | TTFT p99 | ITL p99 | mem GB | L | replicas | MATH-500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| stage_1_3 (seed) | revert | 5.7 | +0.0% | 5419ms | 175ms | 95.6 | 30 | 730 | 0.7100 |
| prefix_caching | KEEP | 11.8 | +107.0% | 5939ms | 177ms | 93.3 | 30 | 353 | 0.7100~ |
| max_model_len_rightsize | revert | 11.8 | +107.0% | 3185ms | 164ms | 93.2 | 30 | 353 | 0.7100~ |
| chunked_prefill | revert | 11.8 | +107.0% | 5319ms | 176ms | 92.8 | 30 | 353 | 0.7100~ |
| chunked_prefill | revert | 11.8 | +107.0% | 6483ms | 180ms | 93.4 | 30 | 353 | 0.7100~ |
| spec_decode_ngram | revert | 44.3 | +677.2% | 3336ms | 181ms | 93.3 | 30 | 94 | 0.7100~ |
| spec_decode_ngram | KEEP | 54.4 | +854.4% | 2650ms | 176ms | 93.3 | 30 | 77 | 0.7100~ |
| spec_decode_depth | revert | 46.7 | +719.3% | 3128ms | 180ms | 93.3 | 30 | 90 | 0.7100~ |
| spec_decode_depth | revert | 53.7 | +842.1% | 2716ms | 176ms | 93.3 | 30 | 78 | 0.7100~ |
| graph_capture | revert | 37.2 | +552.6% | 2405ms | 174ms | 91.8 | 30 | 112 | 0.7100~ |
| lossless_complete | revert | 54.1 | +849.1% | 2674ms | 177ms | 94.7 | 30 | 77 | 0.7400 |

~ = quality inherited from the baseline. A lossless node cannot move quality by construction, so it is not re-measured.

Result: 5.7 → 54.4 tok/s goodput, 9.5x, with MATH-500 flat at 0.71. Speculative decoding supplies almost all of it.

# Experiment 2 — Qwen3-14B, lossy ladder on MATH-500

eval_repro.py, PINNED at L=30, 100 problems × 3 repeats.

| variant | stage | MATH-500 | spread | goodput tok/s | goodput req/s | thru tok/s | thru req/s | TTFT p99 | ITL p99 | SLO | L | GB | replicas |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| stock | baseline | 0.7333 | 0.0200 | 11.9 | 0.07 | 165.4 | 0.69 | 5300ms | 174ms | 5% | 30 | - | 350 |
| lossless | lossless | 0.7300 | 0.0300 | 47.7 | 0.38 | 164.3 | 0.95 | 2783ms | 178ms | 28% | 30 | - | 88 |
| fp8 | lossy | 0.7500 | 0.0200 | 25.4 | 0.13 | 305.3 | 1.35 | 2618ms | 102ms | 10% | 30 | - | 164 |
| autoquant@6.0 | lossy | 0.7233 | 0.0100 | 33.9 | 0.20 | 379.4 | 2.00 | 2771ms | 83ms | 10% | 30 | 12.0 | 123 |
| autoquant@5.15 | lossy | 0.6800 | 0.0200 | 265.5 | 1.18 | 455.8 | 2.00 | 1224ms | 75ms | 59% | 30 | 10.6 | 16 |
| nvfp4 | lossy | 0.7200 | 0.0600 | 312.6 | 1.40 | 458.0 | 2.00 | 1360ms | 72ms | 69% | 30 | 10.6 | 14 |
| w4a16 | lossy | 0.7067 | 0.0300 | 11.5 | 0.04 | 344.6 | 1.60 | 6878ms | 96ms | 2% | 30 | 10.6 | 362 |

The n=500 re-run that changed a verdict

| variant | stage | MATH-500 | spread | goodput tok/s | goodput req/s | thru tok/s | thru req/s | TTFT p99 | ITL p99 | SLO | L | GB | replicas |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| stock n=500 | baseline | 0.6633 | 0.0020 | 12.0 | 0.07 | 167.6 | 0.71 | 5407ms | 173ms | 5% | 30 | - | 348 |
| nvfp4 n=500 | lossy | 0.6380 | 0.0060 | 272.2 | 1.20 | 457.2 | 2.00 | 1406ms | 74ms | 60% | 30 | - | 16 |

At n=100 NVFP4 read 0.7200 against stock's 0.7333 — a 0.0133 gap against a 0.0600 spread, which is not a result. At n=500 the resolution tightens to 0.0060 and the gap is −0.0253: NVFP4 costs real accuracy on MATH-500. Both absolute scores also drop ~7 points, because the loader takes the FIRST n rows and MATH-500's first 100 are easier than the set as a whole (mean level 3.21 vs 3.44). Absolute n=100 figures are optimistic; paired comparisons on identical problems remain valid.

# Experiment 3 — Qwen3-14B, the same ladder on MBPP+

eval_repro.py, pinned at L=30, all 378 problems × 3 repeats, pass@1 by executing generated code.

| variant | stage | MATH-500 | spread | goodput tok/s | goodput req/s | thru tok/s | thru req/s | TTFT p99 | ITL p99 | SLO | L | GB | replicas |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| stock | baseline | 0.7143 | 0.0053 | 5.8 | 0.02 | 168.9 | 0.76 | 5301ms | 171ms | 2% | 30 | - | 723 |
| lossless | lossless | 0.7169 | 0.0053 | 44.1 | 0.33 | 163.9 | 1.00 | 2996ms | 178ms | 25% | 30 | - | 95 |
| fp8 | lossy | 0.7125 | 0.0053 | 45.5 | 0.22 | 301.9 | 1.35 | 2836ms | 103ms | 16% | 30 | - | 92 |
| autoquant@6.0 | lossy | 0.7099 | 0.0132 | 46.4 | 0.24 | 382.2 | 2.00 | 2250ms | 85ms | 12% | 30 | 12.0 | 90 |
| autoquant@5.15 | lossy | 0.7072 | 0.0106 | 212.5 | 0.98 | 454.0 | 2.00 | 1347ms | 72ms | 49% | 30 | 10.6 | 20 |
| nvfp4 | lossy | 0.7116 | 0.0159 | 277.5 | 1.22 | 456.7 | 2.00 | 1398ms | 73ms | 61% | 30 | 10.6 | 15 |
| w4a16 | lossy | 0.7222 | 0.0000 | 11.5 | 0.04 | 344.5 | 1.60 | 6852ms | 96ms | 2% | 30 | 10.6 | 362 |

MBPP+ separates nothing: repeat spread is 0.0053–0.0159 and the largest deviation from stock is 0.0079, three problems out of 378 — including for NVFP4, which MATH-500 shows to be a real loss. The benchmark is working; it is measuring something with little room to move. Measured on this rig MBPP+ prompts average 94 tokens and completions 37, so a quantization error has very few tokens over which to compound. Use MATH-500 at n=500 to decide whether a quantization is safe; MBPP+ confirms code generation still works, which is a weaker claim.

# Experiment 4 — Qwen3-30B-A3B (MoE), lossless search

run.py optimize, swept, 12 launches, 164 minutes.

| node | kept | goodput tok/s | vs seed | TTFT p99 | ITL p99 | mem GB | L | replicas | MATH-500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| stage_1_3 (seed) | revert | 29.7 | +0.0% | 513ms | 48ms | 95.2 | 2 | 141 | 0.7400 |
| prefix_caching | KEEP | 35.4 | +19.2% | 484ms | 48ms | 91.7 | 2 | 118 | 0.7400~ |
| max_model_len_rightsize | revert | 35.5 | +19.5% | 496ms | 48ms | 93.6 | 2 | 118 | 0.7400~ |
| chunked_prefill | revert | 35.5 | +19.5% | 494ms | 48ms | 92.7 | 2 | 118 | 0.7400~ |
| chunked_prefill | revert | 35.6 | +19.9% | 493ms | 48ms | 90.8 | 2 | 117 | 0.7400~ |
| max_num_batched_tokens | revert | 0.0 | -100.0% | infms | infms | 0.0 | None | None | 0.7400~ |
| max_num_batched_tokens | revert | 15.5 | -47.8% | 826ms | 78ms | 89.7 | 4 | 269 | 0.7400~ |
| spec_decode_ngram | KEEP | 66.6 | +124.2% | 470ms | 105ms | 92.7 | 8 | 63 | 0.7400~ |
| spec_decode_ngram | revert | 47.9 | +61.3% | 399ms | 81ms | 90.6 | 4 | 87 | 0.7400~ |
| spec_decode_depth | revert | 60.0 | +102.0% | 497ms | 159ms | 91.5 | 8 | 70 | 0.7400~ |
| spec_decode_depth | revert | 52.0 | +75.1% | 612ms | 161ms | 92.0 | 16 | 80 | 0.7400~ |
| graph_capture | revert | 48.0 | +61.6% | 583ms | 132ms | 90.9 | 8 | 87 | 0.7400~ |
| lossless_complete | revert | 57.7 | +94.3% | 468ms | 156ms | 90.6 | 8 | 73 | 0.7400 |

Stock vLLM, for reference

| variant | stage | MATH-500 | spread | goodput tok/s | goodput req/s | thru tok/s | thru req/s | TTFT p99 | ITL p99 | SLO | L | GB | replicas |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| stock | baseline | 0.7400 | 0.0200 | 3.8 | 0.02 | 74.8 | 0.36 | 1129ms | 102ms | 6% | 8 | - | 1096 |

Measured at L=8 to match the optimized config's operating point, stock reaches 3.8 tok/s goodput at 6% SLO attainment — its TTFT p99 of 1129 ms is over twice the 500 ms budget. IMPORTANT: this is stock at ONE point, not stock at its own peak. Stock differs from the search seed in three ways (prefix caching ON, CUDA graphs ON, max_model_len 40960 rather than 7168), and the last of those leaves far less KV headroom. A fair stock figure needs its own sweep, which the harness does not currently do for a fixed config.

Capacity of the chosen config — its own sweep

| L | goodput tok/s | TTFT p99 | SLO attainment |  |
| --- | --- | --- | --- | --- |
| 4 | 48.4 | 326ms | 100% |  |
| 8 | 65.6 | 418ms | 100% | PEAK |
| 16 | 57.3 | 539ms | 80% |  |
| 32 | 22.9 | 628ms | 39% |  |
| 64 | 0.0 | 718ms | 0% |  |
| 128 | 1.5 | 1238ms | 0% |  |
| 256 | 0.0 | 1359ms | 0% |  |

A true interior peak at L=8, with 100% SLO attainment at both L=4 and L=8. The SEED's curve is different and must not be substituted: it peaks at L=2 with 41.8 and collapses to 7.2 by L=8. Speculative decoding did not merely raise goodput at a fixed point, it moved the whole curve — which is why the operating point follows the incumbent through the search.

# Pareto frontiers

The non-dominated set over every measurement taken, reverted configurations included. A reverted config is still an operating point someone may want — "less goodput, better latency" is a trade, not a failure. Axes: goodput (max), quality (max), TTFT p99 (min), memory (min).

Qwen3-14B (lossless search)

| node | goodput tok/s | TTFT p99 | ITL p99 | mem GB | L | replicas | MATH-500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| spec_decode_ngram | 54.4 | 2650ms | 176ms | 93.3 | 30 | 77 | 0.7100 |
| lossless_complete | 54.1 | 2674ms | 177ms | 94.7 | 30 | 77 | 0.7400 |
| graph_capture | 37.2 | 2405ms | 174ms | 91.8 | 30 | 112 | 0.7100 |

Qwen3-30B-A3B (lossless search)

| node | goodput tok/s | TTFT p99 | ITL p99 | mem GB | L | replicas | MATH-500 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| spec_decode_ngram | 66.6 | 470ms | 105ms | 92.7 | 8 | 63 | 0.7400 |
| spec_decode_depth | 60.0 | 497ms | 159ms | 91.5 | 8 | 70 | 0.7400 |
| lossless_complete | 57.7 | 468ms | 156ms | 90.6 | 8 | 73 | 0.7400 |
| spec_decode_ngram | 47.9 | 399ms | 81ms | 90.6 | 4 | 87 | 0.7400 |
| max_num_batched_tokens | 15.5 | 826ms | 78ms | 89.7 | 4 | 269 | 0.7400 |

# The two models against each other

|  | Qwen3-14B | Qwen3-30B-A3B | ratio |
| --- | --- | --- | --- |
| parameters | 14.8 B dense | 30.5 B / 3.4 B active | 2.1x total, 0.23x active |
| weights resident | 29.5 GB | 61.1 GB | 2.1x |
| decode roofline | 108 ms | 24.5 ms | 4.4x better |
| measured ITL p99 (seed) | 175 ms | 48 ms | 3.7x better |
| seed goodput | 5.7 | 29.7 | 5.2x |
| seed TTFT p99 | 5419 ms | 513 ms | 10.6x better |
| best after search | 54.4 | 66.6 | 1.22x |
| gain from the search | 9.5x | 2.24x | — |
| sustainable L | 30 | 8 | — |

The MoE starts 5.2x ahead and finishes only 1.22x ahead. Lossless optimization is worth far more on the worse-starting configuration — an argument for running the search on whatever you actually deploy, and against assuming a better model needs less tuning.

The ITL prediction held. A roofline estimated the MoE's decode floor at 24.5 ms against the 14B's 108 ms, a 4.4x edge; measured ITL was 3.7x better, with both models sitting ~1.9x off their own floor.

The prefill prediction held too, and it is the binding constraint. A 620-token prompt routes across effectively all 128 experts (the chance any expert is missed by all 620 tokens is 4e-18), so prefill reads the full 61 GB and the TTFT floor RISES from 108 ms to 224 ms. The MoE's decode advantage is real but partly stranded: ITL uses 19% of its budget while TTFT gates admission.

chunked_prefill did NOT help the MoE (+0.0%), against expectation. Chunking reorders prompt processing; it does not reduce expert weight traffic, which is what makes this prefill expensive.

# Caveats

RULER IS EXCLUDED because the probe is broken, not because the models failed it. It scored 0.05 on the MoE and then MOVED to 0.11 across lossless_complete — a lossless node changes launch flags only and cannot move quality, so that delta is instrument, not model. The corpus itself checks out (all 200 prompts contain all four needles and fit the served context), so the fault is downstream, most likely output format. It is unresolved: the traversal does not save generations, so there is no record of what the model actually emitted.

Ladder rows are pinned at L=30 and are therefore a LOWER BOUND, not each variant's peak. The MoE traversal later showed this hardware sustains 8 concurrent requests for that model — so a row measured at 30 reports the concurrency cliff rather than the configuration.

The MoE stock row is one point (L=8), not stock's own peak. Stock runs with max_model_len 40960 against the search seed's 7168, which changes the KV budget substantially, so its optimum may sit elsewhere.

MBPP+ serving numbers were taken while other processes were active on the same machine: stock reads 5.8 tok/s there against 11.9 for the identical config on the MATH-500 ladder. Accuracy is unaffected — it was recomputed from stored generations — but that goodput column should be re-measured before being used for capacity planning.

All results are single-GPU on GB10 with UNIFIED memory, where gpu_memory_utilization is a fraction of system RAM the CPU also competes for. NVFP4 requires Blackwell and will not load on H100.

Quality was measured at concurrency 32. Batch composition changes the floating-point reduction order in attention and GEMM accumulation, which is the leading cause of verdict flips between identical runs; the measured per-token flip rate on this rig is 0.44%.

# Reproducing this

./setup.sh                                     # deps, data, GPU check
python selftest.py                             # instrument checks, no GPU

python run.py optimize --model <id> --trace data/trace_shared.jsonl \
    --ttft-p99 500 --itl-p99 250 --lossless-only --run-dir runs/<name>

./eval_ladder.sh math_500 500 3                # lossy ladder
python summarize.py runs/<dir>
python docs/make_consolidated_report.py        # regenerate this document

Every run writes run_meta.json recording the command, resolved arguments with their defaults, the model and hardware fingerprint, library versions, commit, and whether the working tree was dirty.